Troubleshooting advice

Introduction

Performance assessments in proficiency testing (PT) can be affected by both failures in individual assessments (i.e. analysis errors) or failure to participate in distributions with sufficient frequency. For PT issues related to analysis error please see points 1 – 8 of this document.

- Methods – Ensure that the method used is appropriate for the examination of the sample.

- Interpretation of MPN tables – Ensure that all dilutions have been used to calculate the MPN value and the MPN tables (if used) are interpreted correctly.

- Where three dilutions have been tested, record the number of TBGA/TBX positives for each dilution to give a 3 figure tube combination number. Use the MPN tables included in ISO 7218 and the NRL E. coli generic protocol. Only category 1 or 2 tube combinations are included in the tables and should be reported.

- Where more than three dilutions have been tested, use the Shellfish MPN calculator (available in the EURL E. coli generic protocol) or the MPN calculator available in ISO 7218 (http://standards.iso.org/iso/7218/)  to determine the MPN from all the dilutions tested. Only category 1 or 2 tube combinations should be reported.

- If a tube combination result is an unacceptable combination (category 3), the result is reported as ‘void’.

- Culture media - Check the quality control data for media to ensure that they are within specifications and performing adequately.

- Equipment - Check that the equipment used for the procedures (incubators, refrigerators, measuring instruments) are calibrated and performing adequately.

- Staff training - Check that the staff performing the tests are fully trained and familiar with all the procedural steps.

- Clerical procedures - Check the sample labeling, laboratory numbering and clerical procedures are adequate and that procedures for ensuring that test results are reported accurately and on time.

- Accreditation- Check that quality procedures are documented and adhered to at all times.

- Internal quality controls (IQC) – Ensure adequate controls are in place and follow-up procedures are in place to deal with IQC failures.

Note the value record by the Excel MPN calculator from ISO 7218 give an MPN/g.  Multiply the value by 100 to obtain MPN/100g.